Local Station Monitor Checklist

Name: __________________   Date: ______________

Prep List (Before Visiting Station(s))

[  ] When you have internet access, ensure you have the FieldKit app downloaded and updated, that you have an account set up on the platform, and that you are a member of the appropriate project and have station access.

[  ] Pack a clean rag and a bottle of clean water in order to wipe down the station.

[  ] If available, pack an extra battery pack and a replacement desiccant packet (in a sealed bag) if used

Station Visit Checklist

Visiting multiple stations? We recommend that you use a different list for each station you visit

Location: ________________________________________________

Station Name : ___________________________________________

Type of station: ___________________________________________

[  ] Take a wide-angle photo showing the station and its surroundings. Tip: Stand a few meters away from the station in order to show all its parts (instrumentation, solar panel, etc.).

[  ] Take a close up photo of just the station case while it is closed

Are all the cables plugged in? [  ] Yes [  ] No

[  ] Push the WiFi button on the outside of the station and connect via the app in order to download data.

If you’re not successful downloading data, take a screen capture video of what you are seeing in the app and describe it below.

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

[  ] Open the case and press the center screen button to light up the station screen. Take a photo and record what you see.

Weather stations:

[  ] Take 2-3 photos of the weather instrumentation, making sure to include the rain tipping bucket, wind instrumentation, and the Stevenson screen covering the weather sensor board

Water chemistry stations:

[  ] Remove each probe from the water carefully and take a close up photo of each.

Water level stations:

[  ] Take 1-2 close-up photos of the distance/level sensor

[  ] Take one photo that shows both the sensor and the water surface.

Station Maintenance Checklist:

[  ] Gently wipe down the solar panel, case, and probes

[  ] If you are using desiccant to alleviate humidity, replace the desiccant packet inside the case.